The optimized multiplier finishes the multiplication in n (here =4) clock cycles.
The multiplier considers the final product as the concatenation of C,A and Q.
It cycles through the bits of Q. If Q[i}=0 it keeps it as it is. If =1 it adds M to A. After checking each bit, it right performs operations such that concatenated {C,A,Q} gets right shifted.(c=A[3],A>>,A[0]=Q[3]) Q[0] gets discarded as it has already been cycled through and is no longer required. {C,A,Q} gives the partial product of the entire multiplication at any point. When adding to the partial product A at each stage, since we are adding to A that gets right shifted, we are technically adding M<<i to the partial product.